| Types of Calculation | Highest Reading Sensor Calculation Method (V1) | Unit Circle Calculation Method (V2) | Threshold Detection + Unit Circle Calculation Method (V3) |
| --- | --- | --- | --- |
| Description | Finds the single sensor with the highest pulse count and outputs its fixed physical angle. | Takes the top 4 highest reading sensors and calculates a weighted average vector using unit circle x, y coordinates. | Filters out noise by selecting all sensors above a minimum threshold, calculating direction via combined vector components. |
| Positives | Extremely low computational overhead. Simple to implement and debug. | Higher angular resolution than physical sensor spacing. Smoother transitions between sensors. | Highly resilient to IR pulsation noise. Dynamically adapts to varying number of active sensors. Exponentially smoother data tracking (via Alpha filter). |
| Negatives | Very low resolution (locked to 30 degree increments). Highly susceptible to single-sensor noise and blind spots. | Heavily distorted by pulsed IR fields. Hardcoded to top 4 sensors, skewing vectors if reflection or ambient IR occurs. | Requires fine-tuning of the TSSP_THRESHOLD and alpha parameters. Marginally higher processing time per loop. |